|  | Health & Safety radioactive material risk assessment Form |
| --- | --- |

| Ra No./ERMS Ref: | Date: 31/07/2026 | Version No.: 1.0 | Review Date: 31/07/2027 | Authorised by: Dan Sandiford |
| --- | --- | --- | --- | --- |

STEP 1 – ENTER INFORMATION ABOUT THE ACTIVITY/TASK, ITS LOCATION AND THE PEOPLE COMPLETING THE RISK ASSESSMENT

| Location name: Petrophysics Laboratory, McCoy Building | Building No.: 200 | Room No.: 111 | Date: 31/07/2026 | Assessed by: Jesse Ash | HSR/Employee representative: |
| --- | --- | --- | --- | --- | --- |
| Radioactive material being used: Olympus Handheld XRF Analyser |  | Radioactive material physical state: ☐Gas ☐Liquid ☒Solid |  |  |  |
| Detail the properties of the radioactive material: ☐Activity (Bq): ☐ Activity concentration (Bq/g): ☒Energy (eV): 40kV ☐ Half-life: ☐Decay mode: ☐ Beta ☐ Alpha ☐ Gamma ☒ X-ray ☐ Neutron |  | Shielding requirements of the radioactive material: ☒Nil ☐Perspex ☐Lead ☐Other: |  |  |  |
|  |  | Monitoring requirements of the radioactive material: ☒Personal dosimetry ☐Survey meter ☐Dose meter ☐Other: |  |  |  |
| Licensing requirements: University of Melbourne’s Radiation Management License |  | Disposal Requirements: ☐Dilution and dispersion ☐ Delay and Decay ☐ Concentration and containment |  |  |  |
| Description of the how the radioactive material is being used (the activity/work using the radioactive material) The Olympus Delta X-ray fluorescence spectrometer is a handheld device that contains x-ray tubes which emit x-ray beams during analysis. Analysis can only be performed if the device is powered on and the trigger has been pressed or it has been activated remotely. When the device is activated the x-ray tubes are energized and x-rays are emitted from the analyzer window. Whilst in the lab the device is only to be used within the shielded test stand and activated remotely to prevent radioactive leakage. In this case the rock sample is placed in full contact with the analyzer beam window before the stand lid is closed. Whilst used in ‘handheld’ mode the device must be pressed up against the rock such that the analyzer beam window is entirely contacted. It then must be triggered manually relying on user experience and training to prevent radioactive leakage. To ensure full contact with the window the sample must be at least 38mm in diameter. Furthermore, the sample must only be used on high-density materials to limit material penetration. |  |  |  |  |  |
| Workplace conditions (Describe layout and physical conditions - including access and egress) The device is kept in a locked cabinet in the Petrophysics Laboratory which is only accessible by swipe card to individuals inducted into the lab. The batteries are stored separately in room 405, which is only accessible by key by individuals trained to use the device. When used in the lab, the device is always kept within the test stand and operated remotely (by laptop). The device may also be used in a field setting which will have varied conditions, but likely to be an open outdoor environment with slightly rough, rocky and/or bushy terrain. |  |  |  |  |  |

| List systems of work for the radioactive material: ● Training ● SOPs ● Manufacturer’s information and instructions ● Inspections ● Security requirements ● Licensing ● Emergency situations | Training: All personnel present during the operation of the device are required to have completed the Universities Radiation Training as per TrainME. In addition, all staff working with the device are required to have had a full induction by a registered user. SOP: A detailed Standard Operating Procedure and Risk Assessment is accessible from the petrophysics laboratory webpage, via the QR code attached to the device, and available physically in the laboratory manual. Manufacturer’s Manuals: Relevant information is stored in the ‘XRF’ folder in the petrophysics laboratory. Security Requirements: The device is kept in a locked cabinet in the Petrophysics Laboratory which is only accessible by swipe card to individuals inducted into the lab. The batteries are stored separately in room 405, which is only accessible by key to individuals trained to use the device. Licensing: The device is licensed under the University of Melbourne’s “Radiation Management License” as required by the state of Victoria. Emergency Situations: in the event of a radiological incident, staff are required to: - Raise the alarm - Get help (call emergency services, UoM emergency phone) - Make the area safe (if sage to do so) - Evacuate the area to a predetermined evacuation point - Report incident to local DRSO and UoM RSA as soon as possible |
| --- | --- |
| Is there past experience or background material/requirements with the radioactive material that may assist in the assessment? ● Existing controls ● SOPs ● Standards ● Industry standards ● Incidents & near-hits ● Legislation and Codes ● Training ● Incident Investigation ● Guidance material ● Emergency situations | The systems above were put in place when the device was used as part of the MSCL line where it was in operation for several years. Therefore, there are still existing controls, SOPs and standards. |

Step 2: RISK RATING – RISK MATRIX AND DEFINITIONS

| Likelihood | Consequence |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Insignificant | Minor | Moderate | Major | Severe |
|  | Almost certain | Medium | High | High | Extreme | Extreme |
|  | Likely | Medium | Medium | High | Extreme | Extreme |
|  | Possible | Low | Medium | Medium | High | Extreme |
|  | Unlikely | Low | Low | Medium | High | High |
|  | Rare | Low | Low | Low | Medium | High |

| Likelihood |  | Consequence |
| --- | --- | --- |
| Almost certain – will occur in most circumstances when the activity is undertaken (greater than 90% chance of occurring) |  | Insignificant –First aid treatment, minor injury, no time off work |
| Likely - will probably occur in most circumstances when the activity is undertaken (51 to 90% chance of occurring) |  | Minor – Single occurrence of medical treatment, minor injury, no time off work |
| Possible – might occur when the activity is undertaken (21 to 50% chance of occurring) |  | Moderate – Multiple medical treatments, non-permanent injury, less than 10 days off work |
| Unlikely – could happen at some time when the activity is undertaken (1 to 20% chance of occurring) |  | Major – Extensive injuries requiring medical treatment (e.g. surgery), serious or permanent injury/illness, greater than 10 days off work |
| Rare – may happen only in exceptional circumstances when the activity is undertaken (less than 1% chance of occurring) |  | Severe – Severe injury/illness requiring life support, actual or potential fatality, greater than 250 days off work |

| Risk Rating Priority for Action |  |  |  |
| --- | --- | --- | --- |
|  | Risk acceptance guide | Action | Recommended action time frame |
| Extreme | Not acceptable | Cease or isolate source of risk Implement further risk controls Monitor, review and document controls | Immediate Up to 1 month Ongoing |
| High | Generally (in most circumstances) not acceptable | Implement risk controls if reasonably practicable Monitor, review and document controls | 1 to 3 months Ongoing |
| Medium | Generally (in most circumstances) acceptable | Implement risk controls if reasonably practicable Monitor, review and document controls | 3 to 6 months Ongoing |
| Low | Acceptable | Monitor and review | Ongoing |

| STEP 3 – Identify hazards and associated risk Scores and controls |  |
| --- | --- |
| For each of the following prompts: - Review the prompts/examples for each hazard category that may potentially exist for the activity/task; - Determine and record an inherent risk score using the risk matrix; - In the comments box, describe when and where the hazard is present; - Specify the risk control type, for each current or proposed risk control; - Provide a control description for each current or proposed risk control; - Where proposed risk control(s) have been identified complete a Health & Safety: Action plan; - Determine the residual risk score using the risk matrix. | Hierarchy of Control (Control Type) El – Elimination S – Substitution En – Engineering Is – Isolation G – Guarding Sh – Shielding A – Administrative T – Training In – Inspection M – Monitoring H – Health Monitoring P – PPE |

| Category | Inherent Risk score | Comments (when and where hazard is present) | Control type | Control description (Current and Proposed) | Residual Risk Score |
| --- | --- | --- | --- | --- | --- |
| Purchasing/receipt of radioactive material | Low | The device poses no radioactive hazard whilst powered off therefore there is little to no risk of exposure when receiving a replacement device. Radioactive material is never inserted into an existing device. |  |  |  |
| Use/activity: Lab operation of the device | Medium | Whilst used as a handheld device there is a risk that the user could accidentally place their hand or body part in the path of the x-ray beam. | En, Sh G A T In M | Whilst in the lab the device is only to be used in the test stand and operated remotely which prevents radioactive leakage. Whilst operational, warnings signs will be placed outside the Petrophysics laboratory reducing the number of individuals exposed. Samples permitted for use must have a minimum diameter (38mm) so that they contact the sensor entirely and limit leakage. All users are to complete the University’s radiation training course on TrainME and are to be inducted on how to use the device by a registered user. The device is to be sent back to the manufacturer for inspection every 3 years and checked for leakage every 3 months by a university radiation officer. All users are to wear dosimeter badges to monitor their level of exposure. | Low |
| Use/activity: Field operation of the device | High | Whilst used as a handheld device there is a risk that the user could accidentally place their hand or body part in the path of the x-ray beam. The field is an uncontrolled environment which increases the risk of incidents. | El | Handheld functionalities on the device have been disabled so that it cannot be used in the field. | Low |
| Storage: | Medium | The device poses no radioactive hazard whilst powered off therefore there is little to no risk of exposure whilst it is in storage. However, there is a risk that it could be misused if stolen. | El, En, A | To prevent theft/misuse the device is kept in a locked cabinet in the Petrophysics Laboratory which is only accessible by swipe card to individuals inducted into the lab. The batteries are stored separately in room 405, which is only accessible by key by individuals trained to use the device. | Low |
| Disposal: | Low | The device is fitted with an X-ray tube which only emits radiation whilst powered. | El | The device will be sent back to the manufacturer for disposal. | Low |
| Other: |  |  |  |  |  |

| STEP 4 – ImpleMEntation and consultation process |  |  |  |  |
| --- | --- | --- | --- | --- |
| Determine the person responsible for reviewing and implementing the risk assessment including the identified controls. Ensure a Health & Safety: Action plan has been completed, reviewed and signed off where proposed controls have been identified. Obtain the authorisation of the management representative. Ensure the DRSO has been consulted. Ensure the HSR (if applicable) has been consulted. Ensure the employees undertaking the activity have been consulted. Record below the names of the persons consulted. |  |  |  |  |
| Management representative | Dan Sandiford |  | HSR/Employee representative |  |
| DRSO |  |  | Employee(s) |  |
| Employee(s) | Jesse Ash |  | Employee(s) |  |
| Person Responsible for implementation or escalation |  | Dan Sandiford |  |  |

Extra writing room - use this page to enter extended comments or descriptions

For use in conjunction with the Health & Safety: Risk management requirements and the Health & Safety: Ionising radiation requirements.

For further information contact your Health and Safety Business Partner.