|  | Health & Safety plant risk assessment Form |
| --- | --- |

| Ra No./ERMS Ref: | Date: 31/07/2026 | Version No.: 2.1 | Review Date: 31/07/2027 | Authorised by: Dan Sandiford |
| --- | --- | --- | --- | --- |

STEP 1 – ENTER INFORMATION ABOUT THE ACTIVITY/TASK, ITS LOCATION AND THE PEOPLE COMPLETING THE RISK ASSESSMENT

| Location name: Petrophysics Lab – McCoy Building | Building No.: 200 | Room No.: 111 |  | Date: 31/07/2026 |  | Assessed by: Jesse Ash |  | HSR/Employee representative: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Users of the plant: Staff and students from the Petrophysics Laboratory |  | Plant (Manufacturer’s name and model no.): TCS - Thermal Conductivity Scanner Serial No.: 15 |  |  |  |  |  |  |
| Purpose of plant: The TCS can be used to scan lengths of material to determine its thermal conductivity and thermal diffusivity. It is particularly useful at the rapid analysis of lengths of core. |  | Does the plant require licensing/registration? |  |  | Yes No | Licensing/Registration No.: |  |  |
|  |  | Additional information for pressure vessels. (Refer to AS 4343) |  |  | Design approval no.: |  | Date of manufacture: |  |
|  |  |  |  |  | Class (A, B, C, D E): |  | Number of pressure chambers: |  |
| Description of how plant is used: The core (or other material) is placed onto the track between two standards of known thermal conductivity/diffusivity. The device is remotely activated. The measuring component of the device then run along the underside of the track to measure the temperature differentials along the standards and samples. The measuring component is composed of a cold sensor, a heating infrared lamp and a hot sensor. |  |  |  |  |  |  |  |  |
| Does the operator require a licence or competency? | External licence |  | Specify: |  |  |  |  |  |
|  | Internal competency |  | Specify: The device is only to be used by inducted personnel |  |  |  |  |  |
|  | No specific competency |  |  |  |  |  |  |  |

| Workplace conditions (Describe layout and physical conditions - including access and egress) The TCS is easily accessible on the bench along the left-hand wall of petrophysics lab. There is an entrance/exit directly beside the use area.. |  |
| --- | --- |
| Consider operation outside of normal conditions ● Cleaning ● Non-standard use ● Breakdown & repair ● Maintenance ● Commissioning ● Decommissioning ● Emergency situations | Non-standard use of the device could pose a potential risk. |
| List systems of work for using the plant: ● Training ● SOPs ● Manufacturer’s information and instructions ● Inspections ● Emergency situations | Training, SOPs and manufacturers information. |
| Is there past experience with the activity/task that may assist in the assessment? ● Existing controls ● SOPs ● Standards ● Industry standards ● Incidents & near-hits ● Legislation & Codes ● Training ● Incident Investigation ● Guidance material | Existing controls, SOPs, training and guidance material. |

Step 2: RISK RATING – RISK MATRIX AND DEFINITIONS.

| Likelihood | Consequence |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Insignificant | Minor | Moderate | Major | Severe |
|  | Almost certain | Medium | High | High | Extreme | Extreme |
|  | Likely | Medium | Medium | High | Extreme | Extreme |
|  | Possible | Low | Medium | Medium | High | Extreme |
|  | Unlikely | Low | Low | Medium | High | High |
|  | Rare | Low | Low | Low | Medium | High |

| Likelihood |  | Consequence |
| --- | --- | --- |
| Almost certain – will occur in most circumstances when the activity is undertaken (greater than 90% chance of occurring) |  | Insignificant –First aid treatment, minor injury, no time off work |
| Likely - will probably occur in most circumstances when the activity is undertaken (51 to 90% chance of occurring) |  | Minor – Single occurrence of medical treatment, minor injury, no time off work |
| Possible – might occur when the activity is undertaken (21 to 50% chance of occurring) |  | Moderate – Multiple medical treatments, non-permanent injury, less than 10 days off work |
| Unlikely – could happen at some time when the activity is undertaken (1 to 20% chance of occurring) |  | Major – Extensive injuries requiring medical treatment (e.g., surgery), serious or permanent injury/illness, greater than 10 days off work |
| Rare – may happen only in exceptional circumstances when the activity is undertaken (less than 1% chance of occurring) |  | Severe – Severe injury/illness requiring life support, actual or potential fatality, greater than 250 days off work |

| Risk Rating Priority for Action |  |  |  |
| --- | --- | --- | --- |
|  | Risk acceptance guide | Action | Recommended action time frame |
| Extreme | Not acceptable | Cease or isolate source of risk Implement further risk controls Monitor, review, and document controls | Immediate Up to 1 month Ongoing |
| High | Generally (in most circumstances) not acceptable | Implement risk controls if reasonably practicable Monitor, review, and document controls | 1 to 3 months Ongoing |
| Medium | Generally (in most circumstances) acceptable | Implement risk controls if reasonably practicable Monitor, review, and document controls | 3 to 6 months Ongoing |
| Low | Acceptable | Monitor and review | Ongoing |

| STEP 3 – Identify hazards and associated risk Scores and controls |  |
| --- | --- |
| For each of the following prompts: - Review the prompts/examples for each hazard category that may potentially exist for the activity/task. - Determine and record an inherent risk score by using the risk matrix. - In the comments box, describe when and where the hazard is present. - Specify the risk control type, for each current or proposed risk control. - Provide a control description for each current or proposed risk control. - Where proposed risk control(s) have been identified complete a Health & Safety: Action plan; - Determine the residual risk score using the risk matrix | Hierarchy of Control (Control Type) El – Elimination S – Substitution En – Engineering Is – Isolation G – Guarding Sh – Shielding A – Administrative T – Training In – Inspection M – Monitoring H – Health Monitoring P – PPE |

| Category | Inherent Risk score | Comments (when and where hazard is present) | Control type | Control description (Current and Proposed) | Residual Risk Score |
| --- | --- | --- | --- | --- | --- |
| Can the following items become ENTANGLED with moving parts of the plant, or materials in motion: | Medium | The TCS scanner has a moving component that could potentially entangle hair and clothing. | En, G, Sh | A cardboard shield has been installed in front of the only entrance into the moving component to prevent accidental entanglement. | Low |
| ● Hair ● Jewellery ● Rags ● Gloves ● Clothing ● Other |  |  |  |  |  |
| Can anyone be CRUSHED due to: | Medium | The TCS scanner has a moving component that could potentially crush an entrapped hand or other body part. | En, G, Sh | A cardboard shield has been installed in front of the only entrance into the moving component to prevent accidental entanglement. | Low |
| ● Falling, uncontrolled or unexpected movement of plant or load ● Lack of capacity to slow, stop or immobilise the plant ● Under or trapped between plant and materials or fixed structure ● Contact with moving parts during testing, inspection, maintenance, cleaning, or repair ● Being thrown off ● Tipping or rolling over ● Parts of plant collapsing ● Other |  |  |  |  |  |
| Can anyone be CUT, STABBED or PUNCTURED by coming in contact with: | Low |  |  |  |  |
| ● Moving plant or parts ● Work pieces disintegrated ● Sharp or flying objects ● Work pieces ejected ● Other |  |  |  |  |  |
| SHEARING – Can anyone’s body parts be cut off between: | Low |  |  |  |  |
| ● Two parts of the plant ● A part of the plan and a work piece or structure ● Other |  |  |  |  |  |
| Can anyone be STRUCK by moving objects due to: | Low |  |  |  |  |
| ● Plant or work pieces being ejected or disintegrated ● Uncontrolled or unexpected plant movement ● Mobility ● Other |  |  |  |  |  |
| Can anyone using the plant or in the vicinity of the plant, SLIP, TRIP or FALL due to: | Low |  |  |  |  |
| ● The working environment ● Uneven work surface ● Lack of guardrails ● Poor housekeeping ● Slippery work surfaces ● Other |  |  |  |  |  |
| FRICTION - Can anyone be burnt due to: | Low |  |  |  |  |
| ● Contact with moving parts or surfaces of the plant ● Material handled by the plant ● Other |  |  |  |  |  |
| Can anyone be injured by ELECTRICAL shock or burnt due to: | Medium | The TCS is powered by a standard power cord and plug. If this cord was damaged it could pose an electrical hazard. | Is, T, In, M | The plug and power point are positioned away from any mechanical hazards to limit exposure All cables are to be inspected before being switched on. | Low |
| ● Damaged/poorly maintained leads or switch ● Working near or contact with live electrical conductors ● Water near electrical equipment ● Lack of isolation procedures ● Other |  |  |  |  |  |
| EMERGENCY STOP BUTTONS - can injury from interaction with the plant be caused by: | Low |  |  |  |  |
| ● Lack of prominence of emergency stop ● Emergency stop not red in colour ● Stored energy being released slowly or at a sequent time ● Lack of clarity of emergency stop markings ● Restarting plant by resetting the emergency stop button ● Emergency stop not being fail to safe ● Other |  |  |  |  |  |
| Can anyone be injured by an EXPLOSION of the following items triggered by plant operation | Low |  |  |  |  |
| ● Gas ● Dust ● Vapours ● Liquids ● Other |  |  |  |  |  |
| Can anyone be SUFFOCATED due to: | Low |  |  |  |  |
| ● Lack of oxygen ● Atmospheric contamination ● Other |  |  |  |  |  |
| ENVIRONMENTAL CONDITIONS – Can anyone suffer ill health due to: | Low |  |  |  |  |
| ● Exposure to high temperatures ● Exposure to low temperatures ● Rain ● Sun ● Flora ● Fauna ● Other |  |  |  |  |  |
| HIGH TEMPERATURE or FIRE – Can anyone: | Medium | The TCS contains an electronic infrared lamp for heating samples. This lamp is strong enough to burn combustible materials and skin. | En, Sh T | A protective plastic cover has been attached over the starting position of the lamp so that there can be no contact with the heating element when the device is turned on. Users are instructed never to place combustible materials in or near the lamp and never to touch the lamp | Low |
| ● Come into contact with objects at high temperature ● Be injured by fire ● Other |  |  |  |  |  |
| Can anyone come into contact with FLUIDS or GASES under HIGH PRESSURE due to: | Low |  |  |  |  |
| ● Failure of the plant? ● Misuse of the plant? ● Other |  |  |  |  |  |
| ERGONOMIC (incl manual handling) - Can anyone be injured due to: | Low |  |  |  |  |
| ● Seating design ● Excessive effort ● Poor lighting ● Repetitive body movement or posture ● Poor workplace or plant design ● Controls layout and design ● Lack of consideration for human behaviour causing mental or physical stress ● Other |  |  |  |  |  |
| RADIATION Can anyone be injured/suffer ill health due to: | Medium | The TCS contains an electronic infrared lamp for heating samples. This lamp is strong enough to cause eye discomfort. | En, Sh T | A protective plastic cover has been attached over the starting position of the lamp to reduce luminosity when the device is turned on Users are instructed never to look into the lamp | Low |
| ● Ionising radiation ● Lasers ● Ultraviolet light ● Microwaves ● Radio waves ● Infrared ● Magnetic resonance ● Other |  |  |  |  |  |
| Can anyone be injured or suffer ill health from exposure to OTHER HAZARDS: | Low |  |  |  |  |
| ● Chemicals ● Fumes ● Dusts ● Vibration ● Noise ● Biologicals ● Lighting ● Toxic gases or vapours ● Engineered nanoparticles ● Other |  |  |  |  |  |

| STEP 4 – ImpleMEntation and consultation process |  |  |  |  |
| --- | --- | --- | --- | --- |
| Determine the person responsible for reviewing and implementing the risk assessment including the identified controls. Ensure a Health & Safety: Action plan; has been completed, reviewed and signed off where proposed controls have been identified. Obtain the authorisation of the management representative. Ensure the HSR (if applicable) has been consulted. Ensure the employees undertaking the activity have been consulted. Record below the names of the persons consulted. |  |  |  |  |
| Management representative |  |  | HSR/Employee representative |  |
| Employee(s) |  |  | Employee(s) |  |
| Employee(s) |  |  | Employee(s) |  |
| Person Responsible for implementation or escalation |  |  |  |  |

Extra writing room - use this page to enter extended comments or descriptions

For use in conjunction with the Health & Safety: Risk management requirements and the Health & Safety: Regulated plant requirements.

For further information, refer to https://safety.unimelb.edu.au/safety-topics/management-systems/implement  or contact your Health and Safety Business Partner.